# VOLUME 3

# TECHNICAL SPECIFICATIONS

T.C.

VIZE MUNICIPALITY

Road Pavement Renewal In The Kale Quarter, Hamam Road – Haci Cesmesi Street Project

Technical Specification

- SUBJECT AND SCOPE

This condition includes the project, technical specification, guarantee and acceptance conditions of all building works and manufacturing related to construction Kale Quarter, Hamam Street - Hacı Cesmesi Street Road Coverage Regulatıon Project in the province of Kırklareli Vize.

Administration 	Vize Municipality

The contractor          it refers to the company that takes the tender and will do business.

The work is required for the delivery of the plenary;

-	Realization of road construction materials and workmanship suitable for the project,

2-	GENERAL CONDITIONS

Kale Quarter, Hamam Street - Hacı Cesmesi Street Road Coverage Regulation Project will be done for the removal of the existing road cover, transfer to the casting site, necessary leveling and corrections made and covered with suitable material for the project, transportation, tools, equipment, etc. all expenses and works are the responsibility of the contractor.

3- 	İTEMS OF WORK  / DEFINITIONS OF UNIT PRICE

01- SPECIAL-01 FLOORING COVERING WITH NATURAL SMALL PARQUET STONE (ROAD, SQUARE, PARK, REMOVAL AND SIMILAR PLACES) (80-100 MM, WITH GRANITE GRAINS) (INCLUDING SAND GROUND AND TRANSPORTED STONE TRANSPORT):

Laying of the floor to be covered and laying of 10 cm thick sand, 10 mm x 10 mm x 10 mm sized right-sided and prismatic natural granite paving stone on the sand layer are laid in the pattern and color in accordance with the details in the project, inclination and joint spacing, rocking of the rocks, sand filling of the joints, 1 m² price including loading horizontal and vertical transportation, unloading, all kinds of materials and wastes, labor, tools and equipment expenses, contractor general expenses and profits:

MEASUREMENT: The coating is calculated on the surface project.

02- SPECIAL-02: NATURAL BASALT PARQUET STONE (10 * 10 CM) WITH FLOOR COVERING (ROAD, SQUARE, PARK, REMOVAL AND SIMILAR PLACES) (INCLUDED STONE TRANSPORT):

Laying of the floor to be covered and laying of sand of 10 cm thickness, laying of the basalt crushed paving stone in the size of (10 * 10) cm on the sand layer at the desired inclination and joint distance, filling, sand filling of the joints, sanding of the surface between the joints, unloading, all kinds of materials and wastes, labor, tools and equipment expenses, contractor general expenses and profit including 1 m2 price ::

Measurement: The coating is calculated on the surface Project

03-SPECIAL-03: 10X20X50 CM ANDESITE BORDER SUPPLY SUPPLY AND INSTALLATION (INCLUDING BORDER TRANSPORT):

Installation of natural andesite borders in 10x20x50 cm dimensions in accordance with the project and technique, covering of joints between two borders with 400 dosage cement mortar, all kinds of materials and wastes, loading in workmanship workplace, horizontal and vertical transportation, unloading, tools and equipment expenses, contractor general expenses and 1 m price including profit:,

MEASUREMENT: The curb size is calculated from the project.

04- SPECIAL-04: NATURAL GRANITE PLAK STONE (30 * 30 CM) WITH FLOORING CONSTRUCTION (PATTERNED SURFACE) (INCLUDING TRANSPORTATION):

5 cm thick layer of 400 kg cement on top of the flat concrete surface made in accordance with the Project, Detail and Specification, 5 cm thickness, 30 cm x 30 cm, all kinds of materials and casualties, loading and unloading at work, horizontal and vertical handling, unloading, workmanship, contractor general expenses and profits of the cement grouting, replacement of cracked plates broken during laying, cleaning of floor face from mortar cavities, price of 1 m2, including:

MEASUREMENT: The entire surface covered and the skirtings, if any, are calculated from the project.

05- SPECIAL-05:NATURAL GRANITE GREASE STONE (50 * 25 * 10 CM) SUPPLY AND INSTALLATION (INCLUDING TRANSPORTATION):

In accordance with the project and technique, installation of natural GRANITE grooved stone in 50x25X10 cm dimensions, closing of joints between two grooves with 400 dosage cement mortar, all kinds of materials and wastes, loading in workmanship workplace, horizontal and vertical transportation, unloading, tools and equipment expenses, contractor 1 m price including general expenses and profit:

MEASUREMENT: Calculation of the gutter size is based on the project.

06- SPECIAL-06: CONCRETE TREE GRAIN TECHNICAL SPECIFICATION (80*80*10 CM):

Product measurements are given in the drawings below.

CONCRETE FEATURES:

Microscopically, it should consist of white and dark colored minerals (from natural stones) in the mineralogical direction along with white cement material on the surface.

Dark minerals, gray and black tones, semi-angular or semi-round shaped, about 0.5-4 mm in size, other minerals in white tones should have larger sizes and different surface shapes not exceeding 15 mm compared to darker colors.

colored minerals, gray and black tones, semi-rounded or semi-rounded tiles, and about 0.5 to 4 mm in size. Minerals in other white tones should have larger sizes and different surface shapes, not exceeding 15 mm in comparison to dark colors.

In general, all minerals should resemble”GRANITE" with cement (fine grained and light colored).

Dark and light colored minerals must be in different proportions and exhibit decomposition

After the products are removed from the mold, after 2 days on average they are sandblasted with steel grid sand blasting machine to remove excess cement and other debris on the surface and the surface must be granite appearance by sandblasting with steel grit.

Concrete C 35 or above standards.

Steel Grit SAE No: G 50. After the concrete surface is impregnated with chemical surface hardeners; bright, protective chemical should be applied to the surface.

AGGREGATE:

The mosaic should be suitable for use in construction work in terms of its petrographic and technological characteristics, which are derived from the quarries present in the nature and are homogeneous, resistant to atmospheric effects. The mosaic will be tough, strong, durable, angular.

The general properties and limit values to be searched in the mosaics to be used in the production of each quality concrete will be as follows.

- The loose unit weight will be at least 1150 Kg / m³.

- The strict unit weight will be at least 1350 Kg / m³.

- The apparent specific gravity shall be at least 2600 Kg / m³.

- Coarse particles in the mosaic will be so small that it will not degrade the gradation.

- The fine material (below 0.063 mm) will not exceed 3% by weight.

- Clay and clay pellets will not exceed 1% by weight.

- Other harmful substances (alkalis, mica, covered cilia, soft and stratified pieces, vegetative soil) will not exceed 2% by weight.

- Visible different colored granules in Color Mosaic will not exceed 5% by weight.

CEMENT:

Super White Portland Cement PO 52.5

Chemical Properties of White Portland Cements

Components 			Mass % (moss)

1. Magnesium oxide (MgO)	5

2. Sulfur trioxide (SO3)		4

3. Insoluble residue		5

4. Loss of ignition			5

5. chloride Cl-			0.1

Physical Properties of White Portland Cement

- The whiteness will be at least 70.

- The reaction should not start before 45 minutes and will end in 10 hours at the latest.

- Total volume expansion will not exceed 10 mm.

- The specific surface will be at least 2800 cm2 / g.

Pressure Strength Values of White Portland Cement

The class and type of WPC will provide the following compressive strengths:

Pressure Resistance (MPa) (at least)

2. Day	      7. Day	28. Day

30        	45	   60

07- SPECIAL-07: FLAT BANK TECHNICAL SPECIFICATIONS

CONCRETE FEATURES:

Microscopically, the surface of white cement material together with mineralogical aspects of white and dark colored minerals (from natural stone) will occur. Dark minerals, gray and black shades, square or semi-circular shaped and about half of 0.5-4 mm, Minerals found in other dark shades of white will have the proportions exceeding 15 mm larger sizes and different surface shapes. In general, all minerals, together with cement (fine-grained and light) a "GRANITE" I will resemble. Dark and light minerals will be at different rates and weathering. Goods removed from the mold after approximately 2 days after blasting sandblasted and steel grid surface than cement maker and other debris to be cleared, steel surface by grit blasting will be given the appearance of granite. Concrete will be at 35 C or higher standard.

After the concrete surface impregnated with a chemical surface hardeners; surface gloss, protective chemical application will be made.

AGGREGATE:

Mosaic extracted from existing quarries in nature, homogeneous, resistant to atmospheric effects will be suitable for use in construction works in terms of petrographic and technological capabilities. Mosaic tough, strong, durable, will be cornered.

- General characteristics and limits of the concrete at all the qualifications to be used in the production of mosaic will be as follows. Loose unit weight of at least 1,150 kg / m³ will be.

- Loose unit weight of at least 1,150 kg / m³ will be.

- Apparent specific weight of at least 2600 kg / m³ will be.

- Coarse grains in Mosaic enough to disrupt the graduation will be minimal.

- Fine material (less than 0.063 mm) the amount will not exceed 3% by weight.

- Clay and clay pellets will not exceed 1% by weight.

- Other harmful substances (alkali, mica, clay-covered grains, soft and layered pieces, topsoil) will not exceed 2% by weight.

- By weight of grains in different colors in the visible color mosaic will not exceed 5%.

CEMENT:

Super White Portland Cement PÇ 52.5

Chemical Properties of White Portland Cement

Components%		by weight (maximum)

1. Magnesium oxide (MgO)	5

2. Sulfur trioxide (SO3)		4

3. The insoluble residues           	5

4. Loss on ignition		5

5.Klorür Cl-			0.1

Physical Properties of White Portland Cement

- Whiteness will be a minimum of 70.

- Plugs should start before the end of 45 minutes and will have a maximum of 10 hours.

- Total bulking shall not exceed 10 mm.

- Specific surface at least 2800 cm2 / g.

White Portland Cement Compressive Strength Values

BPC will provide the type of class and compressive strength:

2. days	      7. days	28.days

30        	45	   60

WOOD AND INSTALLATION

I. CLASS Sapelli timber is made of timber, the humidity will be dried automatically in the oven to limit 8-12%. Dried timber, which will be treated in the profile after multiple slicing machine, Laminated wood boards on both sides of the bean calibrated and sanding will be applied. CNC machines will be made on. In profile, the machine will be processed after the final sanding transparent protective water-based wood varnish is applied. Wood will be fixed at 5mm thick 304 stainless blade. Wood Group will be secured with stainless steel tapping screws countersunk slot opened in the concrete of the lamps.

- Glue D / D4

- Humidity: 10% + 2

- Abrasive: 120 Sand

It is a product visual representation. Only the dimensions will be followed. The seating part is 15 cm thick and the wooden covering is 3 cm.

08- SPECIAL-08: TECHNICAL SPECIFICATIONS FOR WOOD-METAL TRASH BIN

Width	: 52 cm.

Depth	: 42 cm.

Height	: 100 cm.

Bin Diameter	: 32 cm. Bin Height	: 40 cm.

- 60x3 mm pipe profiles, 30x60x40 cm concrete will be 	anchorage and the trash can assembly will be done. Detail is shown in the project.

- The trash can assembly will be carried out according to the details of the approved project.

09- SPECIAL-09: COMPOSITE BORDER ELEMENT H 45CM:

Height: 4500 mm

Base Dimension: 250 mm x 250 mm

Weight: 10, 50 Kg

Material Type: Composite

Color Options: Black

Reflective: 20 mm Normal Performance Yellow Reflective

Mounting Shape: 10 x 120 mm Trifon screw and 16 mm plastic Dubel.

It is a product visual representation.

The composite material consists of polyester, glass fiber, filler, and pigment materials. Depending on the product, additional chemical materials can be added. The polyester in the composition gives chemical resistance and binding property to the product and the glass fiber gives mechanical strength. It can be produced in desired color thanks to the pigments to be incorporated into the composite material. Color should be given during production.

Products made from composite materials are extremely resistant to atmospheric conditions, chemical, abrasion and burning. Thanks to the glass fiber it contains, it is resistant to impact, cracking, corrosion and corrosion. Composite material products have insulating properties. Homogeneously produced composite material in several stages

After 15-20 hours of rest, it is ready for printing. Composite material is pressed in hot molds. The mold temperature should be between 125-145 ° C. The press is between 50-250 bar according to the weight and size of the press product. The hardening time of the composite material varies from 180 to 600 seconds depending on the thickness of the product. After the hardening process is completed, the mold is opened and the product is cooled to an appropriate outside location. Various apparatuses and gauges can be used to prevent deformation during cooling. The final shape is obtained by taking the burrs on the product.

4. OTHER CONDITIONS

1.	The General Terms and Conditions of Construction will be followed.

2.	The origins of the project will be taken from time to time and all production will be done according to the approved project.

3.	Workmanship will be first class, technical equipment, professional quality. Full compliance with the standards in the technical specification will be ensured.

4.	Depending on the nature of the work, the contractor company is obliged to submit the details of the necessary manufacturers companies. The contractor shall obtain the consent of the consignee before manufacturing.

5.	Any damages incurred during the course of the work shall belong to the contractor.

6.	Excavation will pour debris etc. where the material municipality will show. There will be no additional charge for this. Excluding the area where the municipality has shown, excavation and debris, etc., will be legal actions.

7.	A new manufacturing process will not be carried out unless the required technical suitability is checked.

8.	The proposal will be made after the work is done. When the bidders submit the bids, they will be deemed to have their place, and then they will not be able to appeal..

9.	In the event of a contradiction in the project, manufacturing will be carried out in accordance with the decision of the control organization.

10.	Workplace safety at the site shall be provided by the contractor. The Contractor shall indemnify the Contractor and third parties for any damages incurred during the operation. The contractor will be responsible for all kinds of Occupational Health and Safety accidents.

11.	It is the responsibility of the contracting officer and the SGK to work in accordance with the Labor Law in force.

12. 	The excavation work to be carried out will be carried out meticulously and the contractor is responsible for all kinds of damages.

13.	The contractor is responsible for all kinds of insurance to be made during shipment.

14. 	Any materials used will be the responsibility of the transporter instead of the construction site and transportation fee will not be paid.

8. PUNİSHMENTS

The criminal provisions specified in the contract specifications and contract provisions shall apply.

9. TECHNICAL DOCUMENTS and CERTIFICATES

It is the responsibility of the contractor to find work safety trainings in which the personnel who will work on the job have job-related professional qualification certificates. It is forbidden to employ personnel who do not have job security and do not have a report that they can work in heavy and dangerous jobs.

10. DELIVERY AND ACCEPTANCE

Acceptance procedure will be done according to the General Conditions of Construction Works.